ML Deployment on Microcontrollers: Workflows and Optimization

The deployment of complex machine learning models, such as neural networks (NNs), onto a resource-constrained microcontroller like the STM32 requires specific stages aimed at optimizing the model size and efficiency to fit within the device's limited flash and RAM memory.

There are two primary prescribed methods detailed in the sources for achieving this: the TensorFlow Lite Micro pipeline and the STM32Cube.AI platform.

1. TensorFlow Lite Micro Pipeline

This method involves several necessary steps, often performed on a PC (Python), to prepare the model for the microcontroller environment:

Step 1: Model Conversion The trained NN model (developed using platforms like TensorFlow or Keras) is first converted into the TensorFlow Lite (TFLite) format. TFLite uses a specific structure called flatbuffer, which is an efficient data format designed to be accessed without requiring the entire file to be loaded into memory.

Step 2: Model Optimization Conversion alone is usually insufficient due to model size; therefore, optimization is necessary. Key optimization techniques include:

- Quantization: This process reduces model size and speeds up computation by converting weights and activations from float32 (floating-point) to lower precision data types, typically int8 (signed 8-bit integers). This can be performed either post-training (e.g., Dynamic Range or Full Integer Quantization) or during training (Quantization Aware Training).

- Pruning: Eliminating weak connections between neurons by setting weights below a threshold to zero, leading to a sparse model structure.

- Weight Clustering: Grouping similar weights and representing these groups using their centroid values, which standardizes weight usage and reduces model size.

Step 3: Conversion to C Array The optimized TFLite model, stored in its binary flatbuffer format, must be converted into a C array (e.g., using utilities like xxd or a Python script like tflite2cc.py) so that it can be compiled and deployed to the microcontroller's memory.

Step 4: Deployment using TensorFlow Lite Micro Library The microcontroller environment uses the TensorFlow Lite Micro library (TFLite Micro), which is a specialized, limited version of TFLite designed specifically for microcontrollers. The resulting C array containing the model parameters is integrated into the IDE project (e.g., STM32CubeIDE or Mbed Studio) along with the TFLite Micro library files for inference execution.

2. STM32Cube.AI Platform Workflow

STMicroelectronics provides the STM32Cube.AI platform (including the web-based Developer Cloud) which automates many of the conversion and optimization steps.

Step 1: Model Loading The original model file (Keras, ONNX, or TFLite format) is fed directly into the STM32Cube.AI Developer Cloud.

Step 2: Quantization The user selects the desired input and output data types (e.g., int8 or float32) for post-training quantization within the platform interface.

Step 3: Optimization The user selects optimization strategies, such as aiming for a balance between RAM size and inference time, optimizing specifically for RAM size, or optimizing for inference time.

Step 4: Code Generation and Deployment After benchmarking the optimized model against the target STM32 board, the platform generates the final deployment files (such as "C Code," an "STM32CubeIDE Project," or "Firmware"). This generated code, along with necessary supplementary libraries (like X-CUBE-AI), is then integrated into the chosen IDE project (STM32CubeIDE or Mbed Studio).

Tensorflow Lite Optimization ways

The deployment of machine learning models onto resource-constrained microcontrollers like the STM32 often requires model optimization, which can be achieved via specialized techniques within the TensorFlow Lite ecosystem. The sources detail three primary methods for TensorFlow Lite model optimization: Quantization, Pruning, and Weight Clustering.

1. Quantization

Quantization is the process of reducing the precision of the model's parameters (weights and activations) to decrease model size and speed up computation. TensorFlow Lite offers several quantization options:

- Dynamic Range Quantization: This method converts model weights from float32 to the lower precision int8 data type. Computations are carried out in int8 to reduce latency during inference, although input and output values remain in float32 format. This approach is often suggested as a first step because it does not require a calibration dataset. It can be enabled by setting converter.optimizations to tf.lite.Optimize.DEFAULT.

- Full Integer Quantization: In this method, the int8 data type is used throughout the entire model's operations. This technique requires a representative dataset for calibration to determine a suitable quantization range. Full integer quantization has two variants:

  - Integer with float fallback: Uses integer quantization internally but keeps input/output values in float32.

  - Integer only: Enforces int8 (or uint8) data types for model weights, all internal operations, and the input/output tensors, providing the maximum size reduction and latency improvement.

- Quantization Aware Training (QAT): Unlike the post-training methods above, QAT is applied during model training. It simulates quantization effects in the forward pass while the backward propagation remains unchanged, allowing the optimizer to minimize the loss and reduce the quantization error simultaneously, thus preserving model accuracy despite the data type reduction.

2. Pruning

Pruning is an optimization technique focused on eliminating unnecessary complexity by introducing sparsity within the model.

- Mechanism: Pruning identifies weak connections between neurons by inspecting the magnitude of the model's weights. Weights found to have a magnitude below a defined threshold are set to zero, effectively breaking these weak connections.

- Effect: This process creates a sparse weight matrix, which, if exploited, can reduce the model's storage requirements and speed up inference by skipping operations involving zero weights.

- Implementation: Pruning parameters, such as initial and final sparsity levels (initial_sparsity and final_sparsity), are configured using a pruning schedule (e.g., PolynomialDecay) before training or fine-tuning the model. Pruned models can then undergo quantization for further size reduction.

3. Weight Clustering

Weight clustering is an approach used to group similar weight values within the neural network model.

- Mechanism: Clustering algorithms (like k-means) are applied to the model's weights. After clustering, each group of similar weights is represented by a single value, known as its centroid.

- Effect: This process standardizes the weight values used across the model, leading to a smaller model size because only the cluster centroids and an index map need to be stored, although this process may introduce a decrease in model accuracy due to the loss of precision.

- Implementation: Weight clustering can be applied to a TensorFlow model by specifying the desired number of clusters (e.g., 16) and the initialization method for the cluster centroids. The clustered model is then fine-tuned (trained for a few more epochs) before being saved. Like pruning, weight clustering can be combined with quantization for additional size benefits.

Classification, regression, and clustering are distinct groups of machine learning methods used to achieve different functions within an intelligent system. These approaches can be differentiated based on their primary function or goal:

| Machine Learning Method | Function (Goal) | Characteristics |
| --- | --- | --- |
| Classification | The function of a classifier is to infer the class label of an unknown observation that is summarized by its features. | This involves partitioning the feature space into a limited number of known classes using a decision boundary. The classifier takes feature values as input and produces a class label as output. Classification problems typically deal with discrete outcomes (e.g., classifying CPU temperature as "hot" or "normal"). |
| Regression | The function of a regressor is to find the relationship between two or more variables. This usually leads to predicting future values based on given data. | This method models the relationship between input variables (features) and continuous output variables, often framed as a function fit problem. Regression is used for tasks like filling missing data, filtering data, or predicting a numerical quantity (e.g., predicting the next hour's temperature). |
| Clustering | The function of a clustering algorithm is to group feature values based on their inherent characteristics. | Unlike classification and regression, clustering is performed without using labeled data. The algorithm takes feature values and the desired number of clusters (in some cases) as input, and it groups the data based on predefined rules, producing cluster centers (centroids) as output. Clustering is useful for identifying inherent groups within data (e.g., treating normal activity as one cluster and a fall action as another for a fall detection system). |

Chapter 5

Chapter 5, "Introduction to Machine Learning," establishes the foundational concepts essential for developing embedded machine learning systems discussed throughout the remainder of the sources,. These core concepts include data generation for testing, formal feature extraction processes, and feature data normalization.

5.1 Random Number Generation (RNG) as Pseudosensor Data

The chapter begins by introducing the use of random number generation (RNG) to create synthetic or pseudosensor data. This is valuable for designing and testing machine learning systems in a fast and reliable manner, particularly when live sensor data acquisition is not feasible.

- RNG Methods: Digital systems typically employ Pseudo-Random Number Generators (PRNGs), which are deterministic algorithms that mimic random behavior based on a starting seed value. Common methods include the Linear Congruential Generator (LCG) and the advanced Mersenne Twister, which is the default RNG used in languages like Python and C,.

- On PC (Python): The NumPy library offers functionality to generate random numbers based on specific Probability Density Functions (PDFs), such as Gaussian (Normal) or Uniform. Examples show generating noisy two-dimensional data suitable for classification tasks and noisy line or sine data for regression applications,,.

- On STM32 (Microcontroller): The STM32 microcontroller includes a dedicated Random Number Generator (RNG) peripheral unit. This unit samples an analog noise source and post-processes it to produce 32-bit random numbers. Libraries like lib_rng are used within STM32CubeIDE or Mbed Studio projects to utilize this functionality,.

- Data Visualization (Histograms): A histogram can be constructed from generated or acquired data samples to form a sample PDF. This involves dividing the data range into bins and counting the samples within each bin. Choosing an appropriate bin size is critical; a small size results in a sparse histogram, while a large size results in a crude one.

5.2 From Sensor Data to Feature Extraction

This section addresses the crucial step of converting raw sensor data into meaningful inputs for machine learning models:

- Definition of Feature: A feature is defined as the raw or processed sensor data fed into the machine learning system.

- Feature Extraction Process: Raw data may be too large or not informative enough (e.g., a single sample from a time series). Feature extraction summarizes raw data using functions or transformations, producing concise and usable feature values. This is essential for various data types, such as processing an array of time-series readings to yield a single representative value, analyzing audio signals, or transforming digital image pixel values,.

- Feature Space Formation: When multiple features are used together, plotting them as coordinates creates a feature space, typically limited to three dimensions for visualization.

5.3 Normalizing Feature Values

Feature values often require preprocessing to prevent issues like noise degradation or feature dominance (where a feature with a larger range overshadows others).

- Min-Max Normalization: This technique scales original feature values ($f$) to fit within the range $$ using the formula $f_{\text{norm}} = \frac{f - f_{\text{min}}}{f_{\text{max}} - f_{\text{min}}}$. However, this method is highly vulnerable to outliers.

- Z-Score Normalization: This method transforms features so they have a zero mean and a unit standard deviation by calculating $f_{\text{norm}} = \frac{f - \mu_f}{\sigma_f}$. This generally places the majority of feature values within the range $[-3, 3]$ and is less sensitive to extreme outliers than min-max scaling, although it assumes the feature values are normally distributed,.

Applications: Feature Extraction Examples

The chapter concludes with detailed examples of extracting features for various embedded applications:

- Accelerometer Data (Human Activity Recognition): Using the WISDM Dataset, raw time-series data is subjected to segmentation (dividing the stream into fixed-size chunks, e.g., 80 samples, with a step distance, e.g., 40),. Features extracted include mean acceleration, the count of positive values, standard deviation in the frequency domain (calculated using FFT), and Signal Magnitude Area (SMA). On the microcontroller, implementation relies on the CMSIS-DSP library for efficient FFT operations.

- Audio Signals (Keyword Spotting): Using the Free Spoken Digit Dataset (FSDD), the core features are Mel-Frequency Cepstral Coefficients (MFCCs). MFCC extraction involves converting the audio signal into a time-varying frequency response (spectrogram using STFT), applying Mel-spaced filter banks, and finally using a Discrete Cosine Transform (DCT) to get coefficients representing spectral changes. This implementation also uses the CMSIS-DSP library on the microcontroller.

- Digital Images (Handwritten Digit Recognition): Using the MNIST Dataset, image features are extracted using Hu Moments. Hu moments are a set of seven values derived from central moments, providing a description of the object's shape that is invariant to translation, scale, and rotation,. The process involves calculating raw moments ($m_{pq}$) and central moments ($\mu_{pq}$). Implementation requires complex C functions for moment calculation on the microcontroller,.

Chapter 6

Chapter 6, titled "Classification," introduces the concept and application of classifiers within machine learning, particularly focusing on traditional methods suitable for embedded systems. The chapter systematically covers the fundamentals of classification, detailed background on four key classifier types, deployment strategies, and practical applications.

6.1 What Is a Classifier?

Classification is a machine learning task where the objective is to infer the class label of an unknown observation based on its features. The classifier receives feature values as input and produces one of a limited number of known class labels as output.

- Training and Decision Boundary: Before inference, the classifier must be trained using labeled data samples. This training process involves distinguishing between classes (e.g., "standing still" and "falling" in a two-class problem) by partitioning the feature space with a decision boundary.

- Classifier Types: Classifiers can be categorized into two groups:

  - Parametric classifiers (e.g., Bayes Classifier, Support Vector Machine when not using the kernel trick) rely on a mathematical function with explicit parameters to represent the decision boundary. They have global representation ability but limited modeling capability due to the predefined model form.

  - Nonparametric classifiers (e.g., k-Nearest Neighbor, Decision Tree) do not use a fixed parametric function for the decision boundary; instead, they rely directly on the training data and neighborhood information, exhibiting stronger modeling capability but lacking true global representation.

- Performance Measures: Performance is typically evaluated during the testing (inference) phase using labeled test data different from the training data. Key metrics derived from True Positives (TP), False Positives (FP), True Negatives (TN), and False Negatives (FN) include:

  - Precision: Measures the ratio of true positive predictions to the total positive predictions ($\frac{TP}{TP + FP}$). It is important when the cost of false positives is high.

  - Recall: Measures the ratio of true positives to the total actual positive cases ($\frac{TP}{TP + FN}$). It is important when the cost of false negatives is high.

  - F1 Score: The harmonic mean of precision and recall, useful for balanced metrics, especially in datasets with class imbalance.

6.2 Bayes Classifier

The Bayes Classifier uses probability theory (specifically the Bayes theorem) and often relies on assuming that likelihood values follow a Gaussian Probability Density Function (pdf).

- Training (Parameter Estimation): Training involves calculating parameters (sample mean vector and covariance matrix) from the labeled training set.

  - The Naive Bayes classifier is the simplest form, assuming features are independent and classification is based solely on the Euclidean distance to class mean values.

  - More complex versions involve calculating distances using the Mahalanobis distance, either assuming a common covariance matrix for all classes (Eq. 6.11) or calculating unique covariance matrices for each class (Eq. 6.12).

- Deployment: The trained model parameters (means, covariances, etc.) are exported from Python using the BayesExporter class in the sklearn2c library into C header and source files. The deployment relies on CMSIS DSP library functions on the STM32 microcontroller for matrix operations during inference.

6.3 k-Nearest Neighbor (kNN) Classifier

The kNN classifier is a nonparametric method that depends directly on the training data, overcoming the limitations imposed by predetermined distribution models (like the Gaussian assumption in Bayes).

- Training: The training step for kNN simply involves keeping all training samples from all classes.

- Inference: For an unknown sample, the algorithm calculates its distance to all stored training samples, selects the $k$ closest neighbors, and assigns the unknown sample to the class that holds the majority among these $k$ neighbors. Because the decision boundary depends entirely on local neighbors, kNN is powerful but lacks clear mathematical representation and generalization capacity away from training data.

- Deployment: Deployment requires generating C files that contain the entire set of training data (samples and their labels) using the KNNExporter class.

6.4 Support Vector Machine (SVM) Classifier

The SVM classifier is designed to find a linear decision boundary (hyperplane) that maximally separates two classes while maximizing the distance (margin) to the nearest training samples (support vectors).

- Non-linear Separation (Kernel Trick): If classes are not linearly separable, SVM uses kernels (e.g., Radial Basis Function or RBF, polynomial) to map the features to a higher dimension where a linear boundary can be found.

- Multi-class Classification: Since SVM is inherently binary, multi-class problems are solved using hierarchical approaches like one-to-rest (M classifiers needed for M classes) or one-to-one ($\frac{M(M-1)}{2}$ classifiers needed).

- Deployment: The trained SVM parameters (coefficients, support vectors, intercepts, and kernel type/parameters) are exported using the SVMExporter class.

6.5 Decision Tree Classifier

The Decision Tree is a nonparametric classifier that recursively partitions the data based on a selected feature and threshold value.

- Training: Training involves successive binary splits from a root node to leaf nodes, aiming to reduce impurity (often measured by entropy or Gini index) at each level. The resulting tree is effectively a set of sequential if-else rules.

- Inference: An unknown sample traverses the tree based on these binary rules until it reaches a leaf node, and the class label associated with that leaf node is the output.

- Deployment: The complex structure of the tree (split features, thresholds, and resulting values for each node) is converted into C code using the DTClassifierExporter class for deployment.

Applications

The chapter concludes with three practical applications demonstrating the classifier implementation using featured extraction methods from Chapter 5:

- Human Activity Recognition via Accelerometer Data: The Bayes classifier is trained on the WISDM dataset using extracted features (mean acceleration, positive counts, FFT standard deviation, etc.) to classify six activities (e.g., walking, jogging, sitting).

- Keyword Spotting from Audio Signals: The kNN classifier is trained on Mel-Frequency Cepstral Coefficients (MFCCs), extracted from the Free Spoken Digit Dataset, to classify spoken digits from zero to nine.

- Handwritten Digit Recognition from Digital Images: The SVM classifier is trained on Hu Moments, extracted from the MNIST dataset images, to recognize handwritten digits from 0 to 9.

Chapter 7

Chapter 7, "Regression," focuses on machine learning techniques used to model relationships between variables, primarily to enable the prediction of continuous future values or for general function fitting based on input data. This chapter covers the theory, implementation in Python, and deployment to the STM32 microcontroller for four major regression methods.

7.1 Defining Regression

Regression aims to find the relationship between two or more variables, allowing information about one variable to be derived from others. This is often viewed as a function fit problem used for tasks like filling missing data, filtering, or predicting future values.

- Regression Types (7.1.1): Methods are grouped into:

  - Parametric Regression: Constructs the relationship using a function with explicit parameters (e.g., $y = w_1 x + b$). These methods offer a global representation property (useful for predicting future values outside the training range) but suffer from limited modeling capability because they are bound by a predefined model structure. Linear and Polynomial regression fall into this category.

  - Nonparametric Regression: Does not rely on a fixed parametric function but depends directly on the structure of the training data. This yields stronger modeling capability but lacks the global representation property. kNN and Decision Tree regression fall into this category.

- Performance Measures (7.1.2): The standard metric for evaluating performance is the Mean Squared Error (MSE). MSE measures the average squared difference between the actual output ($y$) and the predicted output ($g(x)$). It is critical to avoid reducing MSE to an absolute minimum indiscriminately, as this can lead to overfitting, where the model incorporates noise into its prediction.

7.2 Linear Regression

Linear regression models the relationship between variables using a linear function.

- Theoretical Background (7.2.1): The method estimates parameters ($w$ and $b$) by iteratively minimizing the Mean Squared Error (MSE). The solution for the linear regressor parameters can be obtained using matrix notation, leveraging the derivation $ \mathbf{w} = (\mathbf{X}^T \mathbf{X})^{-1} \mathbf{X}^T \mathbf{y}$.

- Performance: While effective for linear data, the linear regressor has limited expressive power and generally fails to predict complex nonlinear functions like sine waves accurately.

- Deployment: The LinearRegressor class from the sklearn2c library is used to train the model in Python. The trained parameters are exported into C header (linear_reg_config.h) and source files,. Deployment to the microcontroller involves using the linear_reg_predict function for inference, and a testing framework is used to compare the microcontroller's output against the Python model's output.

7.3 Polynomial Regression

Polynomial regression represents the relationship using a polynomial function of a chosen degree (e.g., $y = w_2 x^2 + w_1 x + b$),.

- Theoretical Background (7.3.1): This approach incorporates higher powers and products of input features as new features. The linear regression matrix formalism (Equation 7.7) is then applied to these augmented features to estimate parameters.

- Performance: Choosing the polynomial order (deg) is crucial: a low order leads to underfitting, while a very high order leads to overfitting (estimating noise along with the underlying function).

- Deployment: The PolynomialRegressor class is used for implementation. Deployment relies on C files (poly_reg_config.h, poly_reg_config.c), and inference is handled by the poly_reg_predict function. The complexity of the polynomial creates a feature space where the input size for the C function is defined by NUM_INPUTS rather than just NUM_FEATURES.

7.4 kNN Regression

kNN regression is a nonparametric method for sequence prediction.

- Theoretical Background (7.4.1): The prediction for a given input relies on identifying its $k$ nearest neighbors within the training dataset. The output value is then determined by calculating the average of the output values of these $k$ neighbors.

- Characteristics: The averaging can be performed uniformly (all neighbors weighted equally) or weighted (where weights are inversely proportional to distance). If the neighborhood parameter $k$ is set too small (e.g., $k=1$), the prediction tends to be highly quantized and overfit the data. Increasing $k$ produces smoother results.

- Deployment: The model requires storing the entire training set (DATA and DATA_VALUES) on the microcontroller,. Inference uses the knn_reg_predict function.

7.5 Decision Tree Regression

Decision Tree regression is a nonparametric model structured around successive binary decisions.

- Theoretical Background (7.5.1): Training involves recursively splitting the data at nodes based on features and thresholds, aiming to reduce impurity (variance/MSE) at each level,. The prediction output for any sample landing in a final "leaf node" is the average value of all training samples within that leaf.

- Characteristics: The final structure is equivalent to a series of if-else rules. Similar to kNN, depth control is crucial: low depth leads to inaccurate, crude models (underfitting), and excessive depth leads to highly specific rules (overfitting),.

- Deployment: The structural parameters of the tree (split features, thresholds, and corresponding output values) are exported to C files,. Inference is performed via the dt_reg_predict function.

7.6 Application: Estimating Future Temperature Values

This section provides a practical application of regression using the SML2010 dataset, which includes indoor temperature, outdoor temperature, and humidity readings sampled at 15-minute intervals.

- Training (7.6.2): A Linear Regressor model is chosen in the example to predict future temperature values. The process involves sampling the input data (e.g., converting to hourly records) and creating a new feature set where the current prediction is based on the previous five temperature records. The model is then trained to minimize the prediction error (Mean Absolute Error, MAE, is used as a metric in the example).

- Implementation (7.6.3): The final trained model is exported for deployment to the microcontroller following the steps outlined earlier in the chapter.

Chapter 8

Chapter 8, "Clustering," details the concepts and methodologies of unsupervised machine learning, focusing on algorithms suitable for embedded systems like the STM32 microcontroller. The chapter introduces clustering generally and provides in-depth coverage of two primary algorithms: k-Means Clustering and Density-Based Spatial Clustering of Applications with Noise (DBSCAN), followed by practical applications.

8.1 What Is Clustering?

Clustering is a process aimed at grouping feature values based on their inherent characteristics. A key distinguishing factor of clustering from classification and regression (discussed in previous chapters) is that it operates without using labeled data.

- Training/Learning Phase: During training, the user supplies feature values along with the desired number of clusters (for some algorithms). The algorithm processes these inputs based on predefined rules to generate cluster centers (centroids) as its output.

- Embedded Applications: Clustering is useful for embedded systems, particularly for tasks like fall detection. In this context, normal human activity is classified as one cluster, and a fall action as another.

- Operation Modes: Clustering can be performed in batch mode, where data samples are collected over time and processed together to form clusters, or in online mode, where each feature value is processed sequentially, causing clusters to evolve continuously. In inference, the system checks the distance of an observed feature value to the cluster centers to determine its cluster label (e.g., normal activity vs. fall).

8.2 k-Means Clustering

K-means is a well-known and extensively used clustering algorithm based on a simple idea. It is a simplification of the Gaussian Mixture Model (GMM), achieved by assuming an identity covariance matrix, so that only cluster mean vectors (centroids) are estimated.

- Theoretical Background:

  - Batch k-Means (Algorithm 2): This requires all data samples to be available initially. Random cluster centers are assigned, and every unlabeled sample is assigned to the closest centroid. The cluster centroids are then recalculated, and this iterative process continues until centroids stabilize or a maximum iteration count is reached. In the inference step, the cluster centers remain fixed.

  - Online k-Means (Algorithm 3): This processes samples sequentially as they arrive ("on the fly"). Cluster centroids are continuously updated based on each new sample. This makes it robust to shifts in data distribution but vulnerable to dramatic shifts caused by outliers.

- Implementation (Python): The Kmeans class from the sklearn2c library is used. Choosing the right number of clusters (n_clusters) is crucial, as too many clusters can redundantly split the data.

- Deployment: Deployment involves exporting the trained cluster parameters into C header (kmeans_clus_config.h) and source files (kmeans_clus_config.c). These files store the number of clusters, the number of features, and the centroid coordinates. Inference uses the kmeans_clus_predict function, which accepts a Boolean parameter (0 for offline/batch, 1 for online) to determine whether cluster centers should be updated.

8.3 Density-Based Spatial Clustering of Applications with Noise (DBSCAN)

DBSCAN forms clusters based on the parameters defining neighborhood size and minimum neighbor count, eliminating the requirement to define the number of clusters beforehand.

- Theoretical Background: The algorithm defines a core point as any point having at least minPts (minimum number of samples) within a distance ε (epsilon).

  - Directly Density Reachable: A point q is directly density reachable from a core point p if it is within distance ε of p.

  - Density Reachable: A point q is density reachable from a core point p if a chain of core points exists between p and q.

  - Cluster Formation (Algorithm 4): A cluster is formed by a core point and all points density reachable from it. Points not clustered are considered noise (outliers).

- Advantages: DBSCAN does not require the number of clusters a priori and is generally robust to outliers and effective for finding irregularly shaped clusters.

- Deployment: Deployment uses the Dbscan class from the sklearn2c library, which exports parameters like NUM_CORE_POINTS, NUM_FEATURES, NUM_CLUSTERS, and EPS. Inference is performed using the dbscan_clus_predict function.

Applications

Chapter 8 concludes with two applications demonstrating the practical use of these clustering methods:

- Fall Detection System (DBSCAN): This application utilizes DBSCAN for anomaly detection using features extracted from accelerometer data (WISDM dataset). DBSCAN is run on the training data, and samples subsequently labeled as -1 are identified as outliers (anomalies).

- Image Quantization (k-Means): This application uses the online k-means algorithm to perform color image quantization. K-means clusters pixels based on their RGB values, reducing the number of distinct colors in the image to the number of clusters used. The k-means inference function is applied iteratively over all image pixels on the microcontroller.

Chapter 9

Chapter 9, "The TensorFlow Platform and Keras API," introduces the primary software environment used for developing neural network (NN) models for eventual deployment onto microcontrollers.

9.1 Introduction to TensorFlow and Keras

The sources prioritize the TensorFlow platform (developed by Google) over alternatives like PyTorch, primarily because TensorFlow offers a specific version called TensorFlow Lite which is designed for use on microcontrollers.

- TensorFlow Core: TensorFlow is fundamentally designed to allow efficient execution of arithmetic operations and function realizations on data across different dimensions, treating them uniformly as tensors.

- Keras API: The Keras API simplifies the process of forming and training NN models and is included by default within the TensorFlow software (version 2.13 used in the sources).

- Access and Installation: TensorFlow can be installed on a PC via the pip install tensorflow command. Alternatively, users can leverage Google Colab in the cloud, which includes TensorFlow pre-installed and offers time-limited free usage of graphical processing units (GPUs).

9.2 Constant and Variable Declarations

TensorFlow provides specific methods for declaring data elements, regardless of their dimension:

- Constants: Constant values (scalars, vectors, matrices, or N-dimensional tensors) are defined using the tf.constant command. Their data type (dtype) is often inferred from the provided value.

- Variables: Mutable data elements are defined using the tf.Variable function.

9.3 Arithmetic Operations

TensorFlow standardizes arithmetic operations across constants and variables of various dimensions:

- Addition and Subtraction: These operations can be performed using the dedicated functions (tf.add, tf.subtract) or the standard operators (+, -).

- Element-Wise Multiplication: This is executed using the tf.multiply function or the * operator.

- Vector and Matrix Multiplications: Matrix operations are handled by the tf.matmul function, provided the tensor dimensions are compatible.

9.4 Activation Functions

These are nonlinear functions crucial for neurons (the basic units of NNs), defining the relationship between input and output. Key activation functions available under Keras include:

- Hard Limiter: Outputs 1 if input $\geq 0$ and 0 otherwise.

- Sigmoid: Defined as $\frac{1}{1 + e^{-x}}$, resulting in an output between 0 and 1.

- Rectified Linear Unit (ReLU): Defined as $\max(0, x)$.

- Softmax: Typically used in the output layer of classifiers to normalize values, ensuring that the probability outputs sum to 1.

9.5 NumPy and TensorFlow

TensorFlow objects (tf.tensor) and NumPy arrays (ndarray) are designed to be compatible and can be used interchangeably. Tensors can be converted to NumPy arrays using the .numpy method, and NumPy arrays can be converted to tensors using tf.convert_to_tensor.

Applications

The chapter concludes with three practical applications concerning model and data management:

- TensorFlow Datasets and Loading Data: TensorFlow provides access to various datasets via the tensorflow_datasets package and the tfds.load function. Data can also be loaded using Keras utilities, such as tf.keras.datasets.mnist.load_data().

- Load and Store Neural Network Models: To save time, trained NN models can be saved and reloaded. Models can be stored either in the Keras H5 format (a single file) or the TensorFlow SavedModel format (a directory containing the computational graph and weights).

- Converting Neural Network Models from Other Platforms: Models developed using other frameworks (like PyTorch or MATLAB) can be converted to TensorFlow format using the Open Neural Network Exchange (ONNX) open standard. This conversion typically involves exporting the model to the ONNX format first, and then using tools like the onnx2tf library to convert it into the desired TensorFlow model format (SavedModel) and TensorFlow Lite models.

Chapter 10

Chapter 10, "Fundamentals of Neural Networks," introduces the basic building block of neural networks (NNs)—the single neuron—and explains the foundational concepts for its formation, training, and use as a simple classifier or regressor, primarily utilizing the TensorFlow platform and Keras API.

10.1 Background: Structure and Definition

The chapter begins by detailing the fundamental components and mathematical representation of a single neuron.

- Structure of the Single Neuron: The neuron receives inputs ($x_i$), each multiplied by a corresponding weight ($w_i$). These weighted inputs are summed, the bias term ($b$) is added, and the total sum is fed into a nonlinear activation function ($\sigma$), yielding the neuron's output ($y$).

- Mathematical Definition: The input-output relationship is mathematically defined as $y = \sigma(\sum_{i=1}^{K} w_i x_i + b)$. Adjusting the parameters (weights and bias) through training allows the neuron to function as a classifier or regressor. Visualization shows that modifying weights changes the neuron's output curve, while the bias term shifts the output along the axis.

10.2 Forming the Single Neuron in TensorFlow

TensorFlow allows defining the neuron at a fundamental level using constants, variables, and activation functions.

- Class Definition and Initialization: A neuron is defined as a Model class inheriting from tf.Module, where the weight ($\text{w}$) and bias ($\text{b}$) terms are defined as TensorFlow tf.Variable objects, typically initialized randomly.

- Obtaining Output: The neuron's output is obtained by computing the weighted sum and applying the chosen activation function, such as tf.keras.activations.sigmoid, via the class's __call__ method.

10.3 Forming the Single Neuron in Keras

Keras simplifies the process by treating the single neuron as a basic layer.

- Layer Definition and Initialization: The single neuron is defined using the tf.keras.layers.Dense class. The parameters units=1 specifies one neuron, and input_shape defines the input size. Initializers (kernel_initializer for weights and bias_initializer for bias) set the starting values, and the activation parameter sets the nonlinear function. This layer is then contained within a tf.keras.models.Sequential model.

- Obtaining Output: The output for a given input is easily computed using the model's predict function.

10.4 Training the Single Neuron

Training involves adjusting the neuron's parameters (weights and bias) using labeled data to minimize the difference between the desired output and the predicted output.

10.4.2 The Loss Function

The loss function measures the numerical difference between the desired (ground truth) and predicted output values.

- Categorical and Binary Cross-Entropy (Classification):

  - Binary Cross-Entropy (BCE): Used for binary classification (two classes). It measures the cross-entropy between predicted probabilities (often derived using the sigmoid function) and ground truth. In Keras, tf.keras.losses.BinaryCrossentropy is used, optionally with from_logits=True if the raw model scores (logits) are provided.

  - Categorical Cross-Entropy (CCE): Used for multi-class classification. It requires predicted class probabilities to sum to 1, usually achieved using the Softmax activation function. In Keras, tf.keras.losses.CategoricalCrossentropy is used.

- Mean Absolute, Absolute Percentage, and Mean Squared Error (Regression): These metrics measure numerical distance. The Mean Squared Error (MSE) is commonly used, calculating the average squared difference between true and predicted output values.

- Cosine Similarity: Used to compare direction (similarity) between extracted feature vectors, resulting in a value between -1 and 1.

10.4.3 Parameter Updating by Optimizers

Optimizers are iterative methods used to minimize the loss function by adjusting parameters.

- Stochastic Gradient Descent (SGD): The fundamental optimization method. It iteratively updates parameters in the direction opposite to the gradient of the loss function, scaled by a learning rate. TensorFlow's tf.GradientTape is used to automatically calculate these gradients.

- RMSProp: An extension of gradient descent that uses a decaying moving average of partial gradients, overcoming the limitations of static learning rates.

- Adam (Adaptive Moment Estimation): A widely used optimizer that extends RMSProp and SGD by using exponentially decaying moving averages of both the first and second moments of the gradients to adapt the learning rate for each parameter.

10.4.4 Training in TensorFlow and Keras

While TensorFlow requires manual implementation of the training loop (including calculating gradients using tf.GradientTape and manually updating weights), Keras abstracts these steps. In Keras, the developer specifies the model architecture, calls model.compile to define the loss function and optimizer, and executes the training using model.fit, where the backpropagation algorithm runs automatically.

10.5 & 10.6 Neuron as Classifier and Regressor

The single neuron can be applied to practical problems:

- As a Classifier (e.g., Binary Classification): By selecting a suitable activation function like sigmoid in the output layer and using a binary cross-entropy loss, the trained neuron forms a linear decision boundary to separate inputs into two classes (e.g., walking/not walking, zero/not zero).

- As a Regressor (e.g., Linear Prediction): The neuron can model linear relationships in data. If the relationship is complex (e.g., sine wave data), the single neuron generally fails to estimate the non-linear relationship accurately. This demonstrates the limitations that motivate the use of multilayer neural networks (Chapter 11).

Chapter 11

Chapter 11, "Multilayer Neural Networks," expands upon the foundational concepts of the single neuron (Chapter 10) by introducing the powerful structure of multilayer neural networks (NNs) to address the complexity and limitations inherent in single-neuron models.

11.1 Background: Moving Beyond the Single Neuron

The single neuron is limited in its ability to solve complex machine learning problems because it can only form a simple hyperplane (a straight line in 2D feature space) as a decision boundary, limiting its classification performance to linearly separable cases. Furthermore, a single neuron has limited functional capacity as a regressor, making it unable to approximate complex, non-linear functions.

To overcome these limitations, multiple neurons are arranged into layers to form multilayer neural networks.

- Structure of the Multilayer Neural Network: An NN typically consists of an input layer, one or more hidden layers, and an output layer.

  - The input layer's neuron count matches the dimension of the input features.

  - The output layer's neuron count matches the number of classes (for a classifier) or the output dimension (for a regressor).

  - Each neuron applies weights, sums weighted inputs, adds a bias, and passes the result through a non-linear activation function ($\sigma$). Different activation functions can be used across different layers.

- Deep and Shallow Neural Networks: A shallow NN has one or two hidden layers, while a deep NN has several hidden layers. Deep NNs generally require more data and training time but offer greater computational power and modeling capability.

- Fully Connected Neural Networks (Dense Layer): The NNs discussed primarily rely on fully connected layers (or dense layers), where every neuron in one layer is connected to every neuron in the next sequential layer.

11.2 Implementing the Multilayer Neural Network in Keras

Keras simplifies the creation of NNs using the sequential layer structure.

- Forming Layers: Layers are typically defined using the tf.keras.layers.Dense class, specifying the number of neurons (units), activation function (e.g., 'sigmoid'), and input dimensions (input_shape) for the first layer,.

- Sequentially Appending Layers: Layers are stacked using the tf.keras.models.Sequential class. This automatically links the output of one layer to the input of the next. This can be done either by passing a list of layers to the Sequential constructor or by incrementally adding layers using the model.add function.

11.3 Training the Neural Network

NN training uses the same core tools (loss functions and optimizers) as the single neuron (Chapter 10) but adds the essential Backpropagation Algorithm.

- The Backpropagation Algorithm: Backpropagation is an iterative algorithm that uses the chain rule to efficiently calculate the gradients of the loss function with respect to every weight and bias in the network. This allows the network to effectively "backpropagate" the total error calculated at the output layer to all previous layers, enabling the consistent updating of all parameters ($\mathbf{W}$ and $\mathbf{b}$) throughout the entire layered structure,.

- Training in Keras: Keras automatically implements the backpropagation algorithm during training. Developers define the network, call model.compile (to set the optimizer and loss function, e.g., Mean Squared Error (MSE) and Stochastic Gradient Descent (SGD)), and execute training using model.fit,.

11.4 & 11.5 Multilayer NN as Classifier and Regressor

Multilayer NNs are substantially more capable than single neurons, enabling them to handle complex relationships:

- Classifier: NNs can form complex, non-linear decision boundaries that effectively separate classes that are linearly non-separable, improving accuracy over single-neuron models,. For binary classification problems, the output layer typically uses the sigmoid activation function and Binary Cross-Entropy loss.

- Regressor: Unlike a single neuron, a multilayer NN can successfully approximate complex, non-linear functions (like a sine wave), provided the hyperparameters (number of layers, neuron count, and training epochs) are configured appropriately to prevent underfitting or overfitting,,.

End-of-Chapter Applications

The chapter reinforces the utility of multilayer NNs by applying them to the same real-world data problems previously introduced.

- Human Activity Recognition (HAR) via Accelerometer Data: An NN is used to classify all six human activity classes (e.g., walking, sitting, jogging), requiring the output layer to have six neurons and use the softmax activation function with Categorical Cross-Entropy loss,.

- Keyword Spotting from Audio Signals: An NN classifies spoken digits from zero to nine (ten classes total). This employs two hidden layers (100 neurons each, ReLU activation) and a final output layer of ten neurons with softmax activation and Categorical Cross-Entropy loss,.

- Handwritten Digit Recognition from Digital Images: An NN uses Hu Moments as features (Chapter 5) to recognize ten digits (0–9). This multi-class application uses two hidden layers (100 neurons, ReLU) and a softmax output layer. It specifically uses Sparse Categorical Cross-Entropy loss because this loss function handles integer labels directly, rather than requiring conversion to the one-hot encoded format,.

- Estimating Future Temperature Values: An NN is trained on five previous temperature readings (features) from the SML2010 dataset (Chapter 7) to predict future temperatures. The model structure includes two hidden layers and uses the Mean Absolute Error (MAE) loss function and the SGD optimizer,.

Chapter 12

Chapter 12, "Embedding the Neural Network Model to the Microcontroller," details the crucial steps required to deploy a trained neural network (NN) model onto a resource-constrained microcontroller, primarily focusing on the TensorFlow Lite ecosystem and the STMicroelectronics toolchain, STM32Cube.AI.

12.1 TensorFlow Lite

The chapter begins by introducing TensorFlow Lite (TFLite), a specialized version of TensorFlow designed to overcome memory and computation restrictions inherent in embedded systems like microcontrollers.

- TFLite Structure: TFLite uses a limited set of operators and stores models in a specific structure called flatbuffer, which is an efficient, cross-platform serialization library. Flatbuffer allows data to be accessed without full deserialization, meaning the model can be analyzed without being loaded entirely into memory, which is critical for microcontroller operation.

- Model Conversion (Keras/TensorFlow to TFLite): Trained Keras or TensorFlow models can be converted to the TFLite format using the tf.lite.TFLiteConverter in Python, typically allowing for the addition of optimization properties during conversion.

- Interpreting the Model: The TFLite interpreter can be run on a PC to verify how the converted model performs inference on input data, ensuring accuracy is maintained post-conversion.

12.2 Model Optimization via Quantization

Model conversion alone is often insufficient due to memory constraints; hence, optimization methods, particularly quantization, are necessary. Quantization reduces the precision of model parameters.

- Dynamic Range Quantization: This method converts model weights from float32 to the lower precision int8 data type. Computations during inference are performed in int8 to reduce latency, but input and output values remain float32. This optimization is enabled by setting converter.optimizations to tf.lite.Optimize.DEFAULT and is recommended as a first step because it does not require a calibration dataset.

- Full Integer Quantization: This approach uses the int8 data type throughout the entire model (weights, activations, inputs, and outputs). Since all values are represented as integers, a representative dataset is needed for calibration to determine the appropriate quantization range. This method has two variants:

  - Integer with float fallback: Uses integer operations internally but keeps external input/output data in float32.

  - Integer only: Enforces int8 (or uint8) data types for model parameters and for input/output tensors, providing maximal size and latency benefits.

- Quantization Aware Training (QAT): Unlike post-training methods, QAT is applied during model training. It minimizes accuracy loss due to quantization by simulating quantization effects during the forward pass while keeping gradient calculation (backward propagation) unchanged. This lets the optimizer reduce the accumulated quantization error alongside the training loss.

12.3 Model Optimization via Pruning

Pruning is an optimization that minimizes model complexity by introducing sparsity.

- Mechanism: Pruning identifies weak connections (weights with magnitudes below a threshold) and sets them to zero, resulting in a sparse weight matrix. This decreases model size and can speed up inference.

- Implementation: Pruning parameters (like initial_sparsity and final_sparsity) are configured using a schedule (e.g., PolynomialDecay). After training or fine-tuning the pruned model, the tfmot.sparsity.keras.strip_pruning function yields the sparse model for export. Pruning can be combined with quantization (dynamic range or full integer) via TFLite conversion for compound optimization benefits.

12.4 Model Optimization via Weight Clustering

Weight clustering groups similar weight values in the NN model.

- Mechanism: Clustering algorithms (like k-means, similar to those discussed in Chapter 8) group weights, and each group is represented by a single value (its centroid). This standardization reduces model size by requiring only the centroids and an index map to be stored, though it may result in some accuracy loss.

- Implementation: The tfmot.clustering.keras.cluster_weights function is used, specifying parameters such as the desired number of clusters (e.g., 16) and the initialization method for centroids. The clustered model is fine-tuned (trained for a few more epochs) before being stripped of clustering metadata and saved. Weight clustering can also be followed by quantization for further size reduction.

12.5 Embedding the TFLite Model to the Microcontroller

Once the model is optimized and converted to TFLite, it must be prepared for the microcontroller environment.

- Converting to C Array: The binary TFLite model file must be converted into a C array for embedding. This can be achieved using tools like the Linux/MacOS xxd utility or platform-independent Python scripts (generate_cc_arrays.py or tflite2cc.py).

- TensorFlow Lite Micro (TFLite Micro): This limited version of TFLite is specifically designed for microcontrollers. The model (now a C array) and the TFLite Micro library files must be included in the microcontroller development project (e.g., STM32CubeIDE or Mbed Studio).

- Deployment and Testing: Deployment involves using the TFLite Micro C++ functions (LIB_MODEL_Init and LIB_MODEL_Run) within the microcontroller code to initialize the model and perform inference. The results can be transferred back to a PC via serial communication for cross-validation against the original Python model.

12.6 Embedding via STM32Cube.AI

STM32Cube.AI is an alternative platform offered by STMicroelectronics that simplifies the embedding process by handling many steps automatically.

- Usage: The user loads the original TensorFlow model (Keras, ONNX, or TFLite format) onto the STM32Cube.AI Developer Cloud.

- Optimization: The platform offers default post-training quantization, allowing the user to select input/output data types (e.g., int8). It also provides optimization options to balance RAM size versus inference time.

- Generation and Deployment: After optimizing, the user can benchmark the model against the target STM32 board and generate firmware (C code or a complete project) ready for deployment using tools like STM32CubeIDE. This simplifies integrating the AI model into a final embedded application.

The chapter concludes by applying these embedding and optimization procedures to the multi-layer neural network applications introduced in Chapter 11 (Human Activity Recognition, Keyword Spotting, Handwritten Digit Recognition, and Temperature Estimation).

Chapter 13

Chapter 13, "Convolutional Neural Networks (CNN)," introduces the architecture and application of CNNs, particularly focusing on their use in image classification tasks and methods for utilizing pre-trained models via transfer learning.

13.1 The Convolution Operation

CNN models are fundamentally based on the convolution operation.

- Mathematical Definition: The chapter focuses on the two-dimensional convolution operation used for images. Convolution involves filtering an input image ($I$) with a filter ($K$), producing an output ($O$). In CNN literature, the correlation operation (Equation 13.2) is often used as a standard representation for convolution, although true convolution requires rotating the filter (Equation 13.1). Since filter coefficients are learned during CNN training, using the correlation operation does not pose a problem, but it changes the coefficients' learned location.

  - Example: Filters like the Sobel filters ($K_x$ and $K_y$) can be used to detect horizontal and vertical edges in an image.

- Implementation via Single Neuron and Sliding Window: The convolution operation can be implemented using a single neuron where the filter coefficients act as the neuron's weights. The complete filtered image is produced through the sliding window approach, where subsections of the input image are successively fed to the single neuron.

- Convolution Definition under Keras: Keras implements convolution with Equation 13.9, which extends the operation to handle batches of images, multiple input channels (like RGB), and multiple filters. The output of the operation can optionally be passed through a non-linear activation function.

13.2 Forming a CNN Model Under Keras

A CNN model typically consists of feature extraction and classification blocks.

- The Convolution Layer: Defined as tf.keras.layers.Conv2D, this layer has several key parameters:

  - filters: The number of convolution filters (depth of the layer).

  - kernel_size: The spatial size of each filter (e.g., 3x3).

  - strides: The spatial shift applied to the sliding window.

  - padding: Set to 'same' (adds zeros to keep input shape) or 'valid' (produces shrunk output).

  - activation: The non-linear activation function applied to the output.

- The Pooling Layer: Used to downsample the spatial dimensions of the output tensor after convolution. This helps tensors represent more general characteristics and reduces the final feature vector size.

  - Non-global pooling layers (MaxPooling2D and AveragePooling2D) operate using a sliding window defined by pool_size and strides.

  - Global pooling layers (GlobalMaxPooling2D and GlobalAveragePooling2D) perform max or average operations across the height and width dimensions, reducing the output size significantly (e.g., a (1, 480, 640, 3) tensor becomes (1, 3)).

- The Flatten Layer: Represented as tf.keras.layers.Flatten, this layer converts the output tensor from the convolution and pooling layers into a one-dimensional vector to be fed into the classification block.

- Feature Extraction Block: This block uses sequential convolution and pooling layers to automatically extract features, contrasting with the manual extraction methods discussed in Chapter 5.

- Classification Block: This block is typically formed by appending fully connected neural network layers (Dense layers) to the feature extraction block, functioning as a classifier. The final layer usually uses the Softmax activation function to output class probabilities.

- Merging Blocks: The feature extraction and classification blocks are merged using the tf.keras.models.Sequential class to form the complete CNN model.

13.3 Training and Testing the CNN Model

- Training the CNN Model: Training involves setting parameters in both the feature extraction and classification blocks based on the training data. This is done using Keras functions like model.compile (to define loss function, optimizer, and metrics) and model.fit (to execute training over a specified number of epochs).

- Visualizing Trained Filters: Insights can be gained by inspecting the filter coefficients (kernels) learned within a convolution layer after training. Keras functions like model.get_layer and layer.get_weights allow observation of these trained parameters.

- Testing the CNN Model: Testing involves feeding unseen test images to the trained CNN model and observing the classification results. The class with the maximum probability is typically selected as the final predicted label using np.argmax.

13.4 Transfer Learning for CNN

Transfer learning addresses the problem of long CNN training times by reusing the feature extraction block of an existing, pre-trained CNN model.

- Mechanism: An existing pre-trained feature extraction block is selected, and its weights and bias terms are frozen (made non-trainable). A new classification block is appended to the frozen feature extractor, and only this new block is trained on the specific problem dataset. This significantly reduces the number of parameters to be adjusted, leading to faster training.

- Lightweight CNN Architectures: Since microcontrollers (like the STM32) have limited memory, lightweight architectures are preferred for transfer learning:

  - SqueezeNet: Designed for resource-constrained devices, featuring "Fire" modules (squeeze and expand layers) to achieve a small model size.

  - ResNet (Residual Networks): Features skip connections and bottleneck layers for computational efficiency and parameter reduction.

  - EfficientNet: Uses compound scaling to uniformly scale layers, filters, and input size to balance performance and efficiency. Keras offers several variants (B0-B7).

  - MobileNet: A family of lightweight architectures (V1, V2, V3) that rely on depthwise separable convolutions to reduce computational cost for mobile and embedded systems.

  - ShuffleNet: Uses channel shuffling and group convolution to improve information exchange and reduce computational cost.

13.5 Embedding the CNN Model to the Microcontroller

The deployment process follows the generalized method introduced in Chapter 12. The trained CNN model (either custom or via transfer learning) is converted to TensorFlow Lite format. This TFLite model is then converted into a C array for deployment onto the STM32 microcontroller for inference. The process also applies to using the STM32Cube.AI platform. Special attention must be paid to the model input data type, especially if images are acquired from a camera as unsigned integers, ensuring consistency with the model's expected input type.

The chapter concludes with applications demonstrating CNN use for Keyword Spotting from Audio Signals (using spectrograms processed as images) and Handwritten Digit Recognition from Digital Images (using models like ResNet).

Chapter 14

Chapter 14, "Recurrence in Neural Networks," introduces the concept of adding memory to neural network models through feedback mechanisms, primarily focusing on Recurrent Neural Networks (RNN), Gated Recurrent Units (GRU), and Long Short-Term Memory (LSTM) for processing sequential data.

14.1 Recurrence and Memory

Recurrence is essential because neural network models covered previously lack memory, meaning their output depends solely on the present input, disregarding previous inputs or outputs. Adding feedback loops introduces memory capabilities.

- Memory by Single Neuron: A single neuron can be given memory by feeding its output $y(t-1)$ back into its input calculation, defined as $y(t) = \sigma(w_x x(t) + w_{y'} y(t-1) + b)$. When appropriate weights and a hard limiter activation function are used, the neuron can "memorize" an input value (e.g., 1), maintaining that output even after the subsequent input becomes zero.

- Memory by Two Neurons: A single neuron memory cannot be reset once set. A structure using two neurons with cross-feedback loops can overcome this, resembling an SR latch (a digital memory element), where input values can explicitly set or reset the stored output. This confirms that recurrence leads to memory.

14.2 Recurrent Neural Networks (RNN)

The RNN generalizes the concept of memory elements using fixed weights to process sequences of inputs over time.

- General Structure: The input vector $\mathbf{x}(t)$ contributes to the current hidden state $h(t)$, along with the previous hidden state $h(t-1)$. The hidden state update is given by $h(t) = \sigma_h(\mathbf{W}_x x(t) + \mathbf{W}_h h(t-1) + b_h)$. The hidden state typically uses a hyperbolic tangent (tanh) or sigmoid activation function ($\sigma_h$).

- Unfolding the RNN: The model can be conceptualized in an "unfolded" or "unrolled" form, resembling a feedforward neural network structure spread across time steps. Crucially, the same weight matrices ($\mathbf{W}_x, \mathbf{W}_h, \mathbf{W}_y$) are used repetitively across all time steps.

- Training the RNN: Training utilizes a method called backpropagation in time, which extends the standard backpropagation algorithm (Chapter 11) by calculating loss based on output errors across sequential time steps.

- RNN Formation in Keras: The tf.keras.layers.SimpleRNN layer is used to define RNNs, and parameters can be set manually via initializers. The optional property unroll=True is available for deployment purposes.

14.3 Gated Recurrent Unit (GRU)

The GRU is a simplified architecture proposed to address performance issues RNNs face with long data sequences.

- General Structure: GRU modifies the hidden state update by employing two primary gates: the update gate ($z_t$) and the reset gate ($r_t$).

  - The update gate ($z_t$) controls how much information from the previous hidden state $h(t-1)$ carries over to the current hidden state $h(t)$.

  - The reset gate ($r_t$) controls how much the previous hidden state affects the calculation of the new candidate hidden state ($\tilde{h}_t$).

- GRU Formation and Training in Keras: GRU layers are formed using tf.keras.layers.GRU. Training involves adjusting the layer's weights and biases, following the same methodology as RNN training.

14.4 Long Short-Term Memory (LSTM)

LSTM is an earlier and more complex architecture than GRU, specifically designed to handle the long-term dependencies in sequential data.

- General Structure: LSTM uses a cell value ($c_t$) to store information and three distinct gates: the input gate ($i_t$), forget gate ($f_t$), and output gate ($o_t$).

  - The forget gate decides what information to discard from the previous cell value ($c_{t-1}$).

  - The input gate and candidate cell value ($\tilde{c}_t$) determine what new information is added to the cell value $c_t$.

  - The output gate controls how much of the cell value is revealed in the final hidden state $h(t)$.

- LSTM Formation and Training in Keras: LSTM layers are defined using tf.keras.layers.LSTM. Training also uses the backpropagation in time method.

14.5 Regression via Recurrence Models

To perform regression using RNN/GRU/LSTM, sequential input data must be correctly prepared. The Keras utility function tf.keras.utils.timeseries_dataset_from_array helps generate time series datasets suitable for recurrence models. When defining the Keras model, the input shape of the recurrence layer must be set as shape=(time_length, n_input), where time_length is the number of time steps and n_input is the size of the feature vector at each step.

14.6 Image Classification via Recurrence Models

Recurrence models can classify images (e.g., MNIST) by processing the image data as a sequence of vectors. For an image like $28 \times 28$ pixels, it can be treated as 28 sequential inputs (time steps), each being a row vector with 28 features.

14.7 Embedding the Recurrence Model

The embedding process for RNN/GRU/LSTM models largely follows the steps for embedding feedforward networks detailed in Chapter 12. A crucial step involves setting the unroll=True property during Keras layer definition. This unrolls the recurrent structure, making it behave similarly to a feedforward neural network, which is necessary for conversion to the TensorFlow Lite format and subsequent deployment as a C array on the microcontroller.

Applications (14.8, 14.9, 14.10)

The chapter includes applications demonstrating the use of recurrence models:

- Keyword Spotting: Using an LSTM model to learn the temporal relationship between Mel frames (spectrogram features) for audio classification.

- Handwritten Digit Recognition: Using an LSTM model to classify MNIST images by processing each row sequentially.

- Estimating Future Temperature Values: Using a GRU model to estimate future temperatures based on previous readings, relying on the model's ability to learn time relations.