Module Name: Microcontroller Programming

Topics Covered:

ARM Architecture, CMSIS, Timers, RTOS, USB, CAN, I²C, SPI, ADC/DAC, Debug Interfaces, Cortex-M Features, RISC-V Architecture

Section A – ARM & Embedded Systems (30 Questions)

1. What is the feature ARM has used from RISC Architecture?

A. Load and Store

B. Fixed Length of Instructions

C. Both A and B

D. None of the Above

2. What is CMSIS project that focuses on Debug Ports and Debugging?

A. CMSIS-SVC

B. CMSIS-DAP

C. CMSIS-RTOS

D. CMSIS-Core

3. Timers are connected to which bus in ARM Architecture?

A. APB Timer Bus

B. APB Bus

C. PPB

D. AHB Bus

4. Why is there no switching between the states required in Thumb-2 Architecture?

A. Due to conditional branch instructions

B. Instructions associated to conditions

C. Thumb-2 instruction set has both 16-bit and 32-bit instructions

D. None of them

5. What is necessary to drive a multitasking system?

A. RTOS

B. Interrupt-driven approach only

C. Polling approach only

D. None of them

6. Harvard Bus Architecture allows:

A. Instruction fetch

B. Data fetch

C. Both A and B simultaneously

D. None of the above

7. Which one of the following is NOT true with respect to bootloader?

A. Primary bootloader resides in boot ROM

B. Executed on power-up and reset

C. Bootloader is mandatory for microcontroller

D. Bootloader can redirect to main()

8. Which statement is TRUE with respect to Exception List in ARM Cortex-M3/M4?

A. Reset has the highest priority

B. NMI has the highest priority after Reset

C. Only A is true

D. Both A and B are true

9. Which timer is used for OS time keeping?

A. Timer 0

B. PWM

C. SysTick

D. Watchdog Timer

10. Input Capture and Output Compare are applications of which peripheral?

A. ADC

B. TIMER

C. UART

D. PWM

11. Which USB device class is used for HDD, SSD and Thumb Drives?

A. USB Mass Storage Class

B. USB Communication Device Class

C. USB Audio

D. None of the Above

12. Which peripheral maintains date and time in a system?

A. RTC

B. Timer

C. WDG

D. PWM

13. Bit stuffing error occurs after:

A. 5 consecutive bits

B. 6 consecutive bits

C. 4 consecutive bits

D. None of the above

14. Which debugging standards are supported by STM32F4 Discovery on-board debugger?

A. JTAG

B. SWD

C. ST-LINK

D. Both A and B

15. Which one is NOT true about Thumb-2 instruction set?

A. It can be 16-bit or 32-bit

B. Improves code density over ARM

C. Fixed length of 32-bit

D. Improves performance over Thumb

16. What is the role of APB Bridge?

A. Balance speed between AHB and APB

B. APB faster than AHB

C. Connect debugger

D. None of them

17. Feature that distinguishes Cortex-M3 and Cortex-M4:

A. Debug system

B. DSP and Floating-Point unit

C. USB

D. None

18. Which debug interface requires minimum number of wires?

A. SWD

B. JTAG

C. Trace

D. PPB

19. Sensors like 3D-ACC and Gyroscope are known as:

A. Tightly coupled sensors

B. MEMS devices

C. On-board sensors

D. Hardwired sensors

20. Address space in ARM Cortex-M3 is:

A. 2²⁴

B. 2⁶⁴

C. 2¹⁶

D. 2³²

21. Motor speed control uses which timer application?

A. ADC

B. DAC

C. PWM

D. WDG

22. UART start and stop bits are used for:

A. Synchronization without clock

B. Indicating start and end of communication

C. Setting baud rate

D. Both A and B

23. Device query is used in which bus to query address?

A. USB

B. SPI

C. I²C

D. None

24. Number of binary bits at DAC input is called:

A. Resolution

B. Linearity

C. Monotonicity

D. Accuracy

25. Resolution of 12-bit ADC/DAC is:

A. 256

B. 1024

C. 2048

D. 4096

26. Medium of communication in CAN bus is:

A. Half duplex

B. Full duplex

C. Simplex

D. None

27. CAN protocol high-speed and low-speed data rates are:

A. 2 Mbps & 500 kbps

B. 250 kbps & 125 kbps

C. 1 Mbps & 500 kbps

D. 1 Mbps & 125 kbps

28. I²C fast mode and high-speed mode operate up to:

A. 100 kbps & 2.4 Mbps

B. 200 kbps & 2.4 Mbps

C. 400 kbps & 3.4 Mbps

D. 300 kbps & 3.4 Mbps

29. ETM stands for:

A. Embedded Trace Microcell

B. Embedded Trace Macrocell

C. Embedded Trace Machine

D. Embedded Trace Microprogramming

30. A slave in SPI is selected using:

A. 7-bit address

B. 10-bit address

C. Chip Select / Slave Select

D. None

Section B – RISC-V Architecture (10 Questions)

31. Considering RV64 architecture, what will Arr[12] correspond to when Arr → a0?

A. 56(a0)

B. 96(a0)

C. 80(a0)

D. 88(a0)

32. Instruction bge rs1, rs2, L1 branches when:

A. rs1 > rs2

B. rs1 ≥ rs2

C. rs1 = rs2

D. rs1 < rs2

33. Calculate the 2’s complement of 7:

A. 1111 1111 1111 1001

B. 1111 1111 1111 1010

C. 1111 1111 1111 1101

D. 1111 1111 1111 0101

34. The program that initiates a procedure and passes parameters is called:

A. Caller

B. Callee

C. Stack

D. Jump

35. Which instruction transfers control to execution environment and higher privilege mode?

A. Call

B. CSR

C. ECALL

D. SVC

36. Which CSR reports the ISA supported by the hart?

A. marchid

B. mhartid

C. misa

D. mvendorid

37. Which component prioritizes and distributes global interrupts in RISC-V?

A. PLCI

B. PLIC

C. CLINT

D. CLNT

38. ISA in RISC architecture is:

A. Closed

B. Open

C. Semi-open

D. Semi-closed

39. Which of the following is an example of U-format instruction?

A. add

B. addi

C. beq

D. lui

40. RISC architecture follows which byte ordering?

A. Little Endian

B. Big Endian

C. South Endian

D. None

# Answer Key

Q1: C

Q2: B

Q3: A

Q4: C

Q5: A

Q6: C

Q7: C

Q8: D

Q9: C

Q10: B

Q11: A

Q12: A

Q13: B

Q14: D

Q15: C

Q16: A

Q17: B

Q18: A

Q19: B

Q20: D

Q21: C

Q22: D

Q23: C

Q24: A

Q25: D

Q26: A

Q27: D

Q28: C

Q29: B

Q30: C

Q31: B

Q32: B

Q33: A

Q34: A

Q35: C

Q36: C

Q37: B

Q38: B

Q39: D

Q40: A