AI + FreeRTOS Integration Strategy

Instead of thinking:

AI Model

Think of the AI model as a service running inside a dedicated FreeRTOS task.

FreeRTOS Scheduler
                     │
 ┌────────────┬─────────────┬──────────────┐
 │            │             │              │
 ▼            ▼             ▼              ▼
Sensor      AI Task      UART Task     Control Task
Task

The scheduler determines when the AI task executes.

Step 1: Keep AI Independent

The first rule is:

Never mix AI code with application logic.

Instead of this:

while(1)
{
    ReadSensor();
    Normalize();
    RunAI();
    printf(...);
    HAL_Delay(100);
}

Create an AI module:

AI/
│
├── ai_manager.c
├── ai_manager.h
├── ai_network.c
├── ai_network_data.c
└── ai_network.h

The rest of the application should only call:

AI_Init();
AI_Run();

This makes the AI reusable across projects.

Step 2: Initialize AI Before the Scheduler

Initialization should happen only once.

main()
↓
HAL_Init()
↓
SystemClock_Config()
↓
MX_GPIO_Init()
↓
AI_Init()
↓
Create FreeRTOS Tasks
↓
vTaskStartScheduler()

AI_Init() should Never initialize the model inside a task.

Step 3: Create an AI Task

The AI task should do only inference.

Wait for data
↓
Run inference
↓
Publish result
↓
Wait again

Example:

void AITask(void *argument)
{
    SensorData data;
    AIResult result;
    while(1)
    {
        if(xQueueReceive(sensorQueue, &data, portMAX_DELAY))
        {
            result = AI_Run(data);
            xQueueSend(resultQueue, &result, 0);
        }
    }
}

Notice that there are:

- no sensor reads

- no UART printing

- no delays

Its only job is inference.

Step 4: Sensor Task

This task periodically acquires data.

Every 100 ms
↓
Read Sensors
↓
Queue Data

Example:

void SensorTask(void *argument)
{
    SensorData sensor;
    while(1)
    {
        sensor.temperature = ReadTemperature();
        xQueueSend(sensorQueue, &sensor, 0);
        vTaskDelay(pdMS_TO_TICKS(100));
    }
}

Step 5: AI Preprocessing

Inside AI_Run():

Receive Sensor Data
↓
Normalize
↓
Copy to Input Buffer
↓
Run CubeAI
↓
Read Output
↓
Return Result

For example, in Lab 2:

Temperature
↓
Normalization
↓
CubeAI
↓
Cold/Warm/Hot

In Lab 4:

Current
Vibration
↓
Normalization
↓
CubeAI
↓
Reconstructed Values
↓
Return Reconstruction

The application can then compute the reconstruction error.

Step 6: Control Task

The AI task should not make application decisions.

Instead:

AI Result
↓
Control Task
↓
Motor
LED
Buzzer
Relay

Example:

Classification
↓
Hot
↓
Turn Fan ON

Forecasting:

Prediction
↓
Temperature > 45°C
↓
Generate Warning

Autoencoder:

Error > Threshold
↓
Stop Motor

Step 7: UART Task

Printing should always be isolated.

Result Queue
↓
UART Task
↓
printf()

This prevents UART delays from affecting AI execution.

FreeRTOS Architecture for Your Labs

Lab 1 – Regression

Sensor Task
     │
     ▼
 Regression AI Task
     │
     ▼
 UART Task

Lab 2 – Temperature Classification

DHT22 Task
      │
      ▼
Classification AI Task
      │
      ▼
Display Task

Lab 3 – Sensor Forecasting

Temperature Task
       │
       ▼
Sliding Window Task
       │
       ▼
Forecast AI Task
       │
       ▼
UART Task

The preprocessing task maintains the rolling buffer of five samples before notifying the AI task.

Lab 4 – Autoencoder

Current Task
      │
      ▼
Vibration Task
      │
      ▼
Preprocessing Task
      │
      ▼
Autoencoder AI Task
      │
      ▼
Control Task
      │
      ▼
UART Task

Generic AI Task

Regardless of the model, the AI task always follows the same structure:

Receive Input
       │
       ▼
Preprocess Data
       │
       ▼
Copy to Input Tensor
       │
       ▼
Run AI Model
       │
       ▼
Read Output Tensor
       │
       ▼
Post-process Output
       │
       ▼
Send Result

Only the preprocessing and post-processing differ between models.

Suggested Project Structure

Project
│
├── Core
│   ├── main.c
│   ├── freertos.c
│   └── app_tasks.c
│
├── AI
│   ├── ai_manager.c
│   ├── ai_manager.h
│   ├── ai_network.c
│   ├── ai_network_data.c
│   └── ai_network.h
│
├── Drivers
│   ├── dht22.c
│   ├── current_sensor.c
│   ├── vibration_sensor.c
│   └── uart.c
│
└── Inc

AI Manager API

To keep the application independent of the underlying AI library, define a small interface:

void AI_Init(void);
AIResult AI_Run(SensorData input);

Then the rest of the application never needs to know whether the model is a regression network, classifier, forecaster, or autoencoder. Only AI_Run() changes internally.