RTOS Laboratory Exercises (FreeRTOS)

Lab 00 – Sample First Program

Objective

Understand the basic structure of a FreeRTOS application.

Problem Statement

- Create your first FreeRTOS application with a single task.

- The task should toggle an LED every 500 ms.

- Print "FreeRTOS Started" on UART every second.

- Observe task execution using SEGGER SystemView.

Lab 01 – Task Delete & Task Handle

Objective

Learn task creation, task handles, and task deletion.

Problem Statement

- Create two tasks:

  - Task_A

  - Task_B

- Task_A should execute for 10 iterations and then delete itself.

- Task_B should continue running indefinitely and print a message every second.

- Verify the deleted task no longer appears in SystemView.

Lab 02 – Task Polling

Objective

Understand polling-based task synchronization.

Problem Statement

- Create two tasks:

  - Producer Task

  - Consumer Task

- Producer updates a shared flag every 2 seconds.

- Consumer continuously polls the flag.

- Print a message whenever the flag changes.

- Observe CPU utilization and explain why polling is inefficient.

Lab 03 – Task Suspend & Resume

Objective

Learn task suspension and resumption.

Problem Statement

- Create three tasks:

  - Task_A

  - Task_B

  - Controller Task

- Controller suspends Task_B after 5 seconds.

- Resume Task_B after another 5 seconds.

- Observe task states in SystemView.

Lab 04 – Cooperative Scheduler & taskYIELD()

Objective

Understand cooperative scheduling.

Problem Statement

- Configure FreeRTOS in Cooperative Scheduling mode.

- Create two tasks with equal priority.

- Both tasks print their names continuously.

- Use taskYIELD() to allow the other task to execute.

- Observe the difference between cooperative and preemptive scheduling.

Lab 05 – Binary Semaphore

Objective

Understand task synchronization using semaphores.

Problem Statement

- Create two tasks.

- Task_A waits for a Binary Semaphore.

- Task_B releases the semaphore every 2 seconds.

- Task_A prints "Semaphore Received" whenever it acquires the semaphore.

- Observe task state transitions.

Lab 06 – Time-Bounded Semaphore

Objective

Learn semaphore timeout handling.

Problem Statement

- Create two tasks.

- Task_A waits for a semaphore with a timeout of 1000 ms.

- Task_B gives the semaphore every 3 seconds.

- Print whether:

  - Semaphore acquired successfully

  - Timeout occurred

- Explain why time-bounded blocking is useful in RTOS.

Lab 07 – Priority Inversion using Semaphore

Objective

Demonstrate priority inversion.

Problem Statement

- Create:

  - High Priority Task

  - Medium Priority Task

  - Low Priority Task

- Protect a shared UART using a Binary Semaphore.

- Demonstrate how the High Priority Task gets delayed by the Medium Priority Task.

- Capture the execution using SystemView.

- Explain why priority inversion occurs.

Lab 08 – Mutex Solving Priority Inversion

Objective

Understand priority inheritance.

Problem Statement

- Repeat Lab 07.

- Replace Binary Semaphore with a Mutex.

- Observe task scheduling.

- Explain how priority inheritance eliminates priority inversion.

Lab 09 – Queue (Pass by Value)

Objective

Learn queue communication.

Problem Statement

- Create Producer and Consumer tasks.

- Producer sends integers 1–20 through a queue.

- Consumer receives and prints the values.

- Observe queue states using SystemView.

Lab 10 – Queue (Structure by Value)

Objective

Send structures through queues.

Problem Statement

- Define a structure containing:

  - Employee ID

  - Temperature

  - Voltage

- Producer sends the structure through a queue.

- Consumer prints all fields.

- Explain pass-by-value communication.

Lab 11 – Queue (Pass by Reference)

Objective

Learn queue communication using pointers.

Problem Statement

- Dynamically create a structure.

- Send only the pointer through the queue.

- Consumer reads the data using the received pointer.

- Compare pass-by-value and pass-by-reference.

Lab 12 – Direct Task Notification

Objective

Understand direct task notifications.

Problem Statement

- Create two tasks.

- Task_A waits for a notification.

- Task_B sends a notification every second.

- Compare Task Notification with Binary Semaphore.

Lab 13 – UART Polling Queue

Objective

Implement UART communication using polling.

Problem Statement

- Receive UART data using polling.

- Store received characters in a queue.

- Consumer Task reads from the queue and prints the received message.

- Explain limitations of polling.

Lab 14 – UART Interrupt Queue with Deferred Processing

Objective

Implement interrupt-driven UART communication.

Problem Statement

- Configure UART Receive Interrupt.

- ISR should place received characters into a queue.

- A dedicated task reads from the queue.

- Display the received message on UART.

- Explain deferred interrupt handling and why lengthy processing should not be performed inside the ISR.

Lab 15 – Software Timer

Objective

Learn software timers.

Problem Statement

- Create a periodic software timer with a period of 1000 ms.

- Timer callback toggles an LED.

- Create another one-shot timer that prints "One Shot Timer Expired" after 5 seconds.

- Compare software timers with periodic tasks.

Suggested Viva Questions (Applicable to All Labs)

- Why is vTaskDelay() used?

- What is the difference between Ready and Blocked states?

- When does context switching occur?

- What is the difference between a Semaphore and a Mutex?

- Why are queues preferred over global variables?

- What causes priority inversion?

- Why are ISR-safe APIs required inside interrupts?

- What is deferred interrupt handling?

- Why is Task Notification faster than a Binary Semaphore?

- What is the advantage of software timers over dedicated periodic tasks?