This diagram explains a bootloader-based execution flow where your microcontroller boots from internal flash but runs an application stored externally (like SD card / pen drive). Let’s break it down simply step by step.

🔁 Overall Idea

- The MCU cannot directly execute code from SD card/USB

- So it:

  - Boots from internal Flash (bootloader)

  - Copies external program into RAM

  - Switches execution to that program

🧠 Step-by-Step Explanation

1) Boot from Flash (Bootloader starts first)

- When MCU powers ON:

  - It always starts from address 0x00000000 (Flash)

- The vector table in flash contains:

  - Initial stack pointer

  - Reset handler (entry point)

👉 This bootloader is a small program whose job is to load the real application.

2) Initialize hardware + Copy to RAM

- Bootloader:

  - Initializes peripherals (SD card / USB)

  - Reads the application binary from external storage

  - Copies it into SRAM (e.g., 0x20000000)

👉 Now your actual application exists in RAM.

3) Update VTOR (Vector Table Offset Register)

- Cortex-M CPUs use a register called VTOR

- It tells the CPU:

“Where is the vector table located?”

- Initially → points to Flash

- Now → updated to point to RAM (0x20000000)

👉 This is critical because interrupts & reset handler must now refer to the new app.

4) Jump to Application (Reset Handler in RAM)

- Bootloader reads:

  - Stack pointer from RAM vector table

  - Reset handler address from RAM vector table

- Then it:

  - Sets stack pointer

  - Jumps to reset handler

👉 Now execution fully switches to your application in RAM

🔄 Flow Summary

Power ON
   ↓
Flash Bootloader runs
   ↓
Reads app from SD/USB
   ↓
Copies to RAM (0x20000000)
   ↓
VTOR → RAM
   ↓
Jump to application reset handler
   ↓
Application runs 🚀

⚙️ Why This Approach is Used

✅ Advantages

- Run large apps not fitting in flash

- Support firmware updates via SD/USB

- Faster execution (RAM is faster than flash)

- Flexible boot mechanisms

⚠️ Important Points

- RAM is volatile → app must be loaded every boot

- Bootloader must be reliable (critical code)

- Correct VTOR setup is mandatory (else crash)

💡 Real-world Example (Automotive / Embedded)

- ECU firmware update via USB/SD card

- Bootloader loads updated software into RAM

- Validates it (CRC/signature)

- Then executes safely

This second diagram is a cleaner, more “standard” boot flow compared to the SD-card case. Here, everything stays inside the chip—just split between Boot ROM and User Flash.

Let’s walk through it in a way that makes the intent very clear.

🧠 What’s really happening here

You have two programs inside the MCU:

- Boot ROM → fixed, factory or protected bootloader

- User Flash → your actual application

The CPU always starts from Boot ROM, but eventually must hand over control to your application.

🔁 Step-by-Step Flow

1) Boot starts from Boot ROM (0x00000000)

- On reset, CPU fetches:

  - Stack pointer

  - Reset handler
from Boot ROM vector table

👉 At this point, only Boot ROM is “visible” to the CPU.

2) Bootloader tasks execute

Boot ROM performs essential checks, for example:

- Is firmware valid?

- Should we enter update mode?

- Any recovery required?

👉 Think of this as a “gatekeeper” before your app runs.

3) VTOR updated → points to User Flash (e.g., 0x00010000)

Now comes the key idea:

- CPU uses VTOR (Vector Table Offset Register) to know:

“Where should I look for interrupt vectors?”

- Boot ROM changes VTOR:

VTOR = 0x00010000  (start of User Flash)

👉 From now on:

- Interrupts

- Exceptions
will use User Flash vector table, not Boot ROM

4) Jump to User Application

Bootloader now:

- Reads:

  - Stack pointer from User Flash

  - Reset handler from User Flash

- Sets stack pointer

- Jumps to reset handler

👉 Control is now fully transferred to your application

🔄 Simple Mental Model

Power ON
   ↓
Boot ROM runs (safe, fixed code)
   ↓
Checks system / firmware
   ↓
VTOR → User Flash
   ↓
Jump to User App Reset Handler
   ↓
Your Application runs 🚀

🔍 Key Insight (Very Important)

The vector table relocation is the heart of this diagram.

Without updating VTOR:

- Interrupts would still go to Boot ROM

- Your app would behave incorrectly or crash

👉 So switching VTOR = switching “control ownership”

⚖️ Compared to Previous Diagram (SD Card Case)

| Aspect | Boot ROM → Flash | Bootloader → RAM (SD Card case) |
| --- | --- | --- |
| Execution location | Flash | RAM |
| External storage | ❌ No | ✅ Yes |
| Copy required | ❌ No | ✅ Yes |
| VTOR change | Flash → Flash | Flash → RAM |
| Complexity | Low | Higher |

💡 Why this design is used

✅ Advantages

- Safe startup (Boot ROM is protected)

- Supports firmware update modes

- Clean separation:

  - Boot logic

  - Application logic

⚠️ Important

- User app must have valid vector table

- Address alignment must be correct (VTOR rules)

- Bootloader must ensure app integrity

🧩 Real Embedded Insight (from your syllabus angle)

In ARM Cortex-M:

- First entry → Initial SP

- Second entry → Reset Handler

- VTOR must be aligned (typically 128/256 bytes)

Top of Form

Bottom of Form