# Lab Programs

### Lab 1

Regression

y = 2x

Students learn:

- AI workflow

- TensorFlow

- TFLite conversion

- STM32 inference

- Single-input/single-output models

### Lab 2

Temperature Classification

Cold

Warm

Hot

Students learn:

- Multi-class classification

- Softmax outputs

- Probability interpretation

### Lab 3

Sensor Forecasting

Students learn:

- Time-series regression

- Sequence inputs

### Lab 4

Autoencoder Anomaly Detection

Students learn:

- Unsupervised learning

- Reconstruction error

- Threshold-based decisions

# Installation of X-CUBE-AI

Step1: STM32CUBEIDE--> Help--> Configuration Tools --> Embedded Software Packages Manager --> STMicroelectronics TAB --> X-CUBE-AI (v10.2.0 or 10.1.0) Install

Step2: Create a New Project, configure your peripherals required

Click and Select X-Cube-AI

Step 3: Select STMicroelectronics.X-CUBE-AI --> Artificial Intelligence X-CUBE-AI--> Core (Select Check box)

Step 4: Adding the Model/Network to your project

Click on Add Network

Step 5: After adding the Model/Network , click on Show Graph and Verify your Model Inputs and Outputs in NETRON Tool

Step 6: After Validating the Graph, Click on Analyze to analyze your network

Step 7: After Analyzing the report, configure any remaining peripherals and then generate the code.

Step 8: Open the project in STM32CUBEIDE , check if “Middlewares” folder is available in the project

Step 9: Inside main.c add below header files

#include "stdio.h"

#include <time.h>

#include <string.h>

#include "ai_datatypes_defines.h"

#include "ai_platform.h"

#include "dht22_temp_classifier.h"

#include "dht22_temp_classifier_data.h"

#include "dht22_temp_classifier_config.h"

Step 9.1: Create Global Variables

/* USER CODE BEGIN PV */

/* Global handle to reference the instantiated C-model */

static dht22_temp_classifier = AI_HANDLE_NULL;

/**

* @brief Statically allocated buffers.

* Buffers can be dynamically allocated using malloc and size information

* given by the report in ai_network_get_report().

*/

// Chunk of memory used to hold intermediate values for neural network

/* Global c-array to handle the activations buffer */

AI_ALIGNED(32)

static ai_u8 activations[AI_DHT22_TEMP_CLASSIFIER_DATA_ACTIVATIONS_SIZE];

// Buffers used to store input and output tensors

/* Array to store the data of the input tensor */

AI_ALIGNED(32)

static ai_float in_data[AI_ DHT22_TEMP_CLASSIFIER_IN_1_SIZE];

/* or static ai_u8 in_data[AI_NETWORK_IN_1_SIZE_BYTES]; */

/* c-array to store the data of the output tensor */

AI_ALIGNED(32)

static ai_float out_data[AI_ DHT22_TEMP_CLASSIFIER_OUT_1_SIZE];

/* static ai_u8 out_data[AI_NETWORK_OUT_1_SIZE_BYTES]; */

/* AI buffer IO handlers */

// Initialize wrapper structs that hold pointers to data and info about the

// data (tensor height, width, channels)

/* Array of pointer to manage the model's input/output tensors */

static ai_buffer *ai_input;

static ai_buffer *ai_output;

/* USER CODE END PV */

# Step 10: Create AI_Init()

Copy the initialization function exactly as shown in your document:

static void AI_Init(void)

{

ai_error err;

const ai_handle acts[] = { activations };

/* Create and initialize the c-model */

err = ai_dht22_temp_classifier_create_and_init(

&dht22_temp_classifier,

(const ai_handle[]){ activations },

NULL);

if (err.type != AI_ERROR_NONE)

{

Error_Handler();

}

/* Reteive pointers to the model's input/output tensors */

ai_input = ai_dht22_temp_classifier_inputs_get(dht22_temp_classifier, NULL);

ai_output = ai_dht22_temp_classifier_outputs_get(dht22_temp_classifier, NULL);

}

# Step 11: Create AI_Run()

Copy the inference function:

static float AI_Run(float input_value)

{

ai_i32 batch;

/* Copy input data to the network input buffer */

((float *)ai_input[0].data)[0] = input_value;

/* Run inference */

batch = ai_dht22_temp_classifier_run(dht22_temp_classifier, ai_input, ai_output);

if (batch != 1)

{

Error_Handler();

}

/* Return the prediction */

return ((float *)ai_output[0].data)[0];

}

Step 12: Initialize the AI Model

Inside main():

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

AI_Init();

# Step 13: Test the Model

Use the example test values from the document:

float test_inputs[] = {1, 2, 3, 5, 7, 10};

for (int i = 0; i < 6; i++)

{

float result = AI_Run(test_inputs[i]);

printf("Input = %.2f --> Prediction = %.2f\r\n",

test_inputs[i],

result);

HAL_Delay(1000);

}

This prints the model predictions over UART.

# Step 14: Redirect printf() to UART

Add:

int _write(int file, char *ptr, int len)

{

HAL_UART_Transmit(&huart4, (uint8_t *)ptr, len, HAL_MAX_DELAY);

return len;

}

Now every printf() is sent to the serial terminal.

# Step 15: Build & Flash the Code

Click

Project

↓

Build Project

↓

RUN

# Step 16: Open Serial Terminal

Open

PuTTY

Set:115200 baud

↓

Terminal

# Step 17: Expected Output

You should see output close to:

1.00 --> 2.01

2.00 --> 3.99

5.00 --> 10.02

7.00 --> 13.98

10.00 --> 20.03

15.00 --> 29.96

Lab-1: AI Inference on STM32 using a Simple Regression Model.

# Objective

The model has learned:

[
y = 2x
]

Example:

Input : 7

Output : ≈14

The STM32 program should:

User Input

│

Normalize (if required)

│

AI Model

│

Output

│

Print Result

# Assumptions

Assume CubeAI generated

multiply_model.c

multiply_model.h

multiply_model_data.c

multiply_model_data.h

and the generated network name is

multiply_model

(Replace the names if CubeAI generated different identifiers.)

# Complete STM32 Logic

## Global Variables

#include "multiply_model.h"

#include "multiply_model_data.h"

AI_ALIGNED(32)

static ai_u8 activations[AI_MULTIPLY_MODEL_DATA_ACTIVATIONS_SIZE];

static ai_float in_data[AI_MULTIPLY_MODEL_IN_1_SIZE];

static ai_float out_data[AI_MULTIPLY_MODEL_OUT_1_SIZE];

ai_handle network = AI_HANDLE_NULL;

ai_buffer *ai_input;

ai_buffer *ai_output;

## Initialize Network

void AI_Init(void)

{

ai_error err;

const ai_handle acts[] = { activations };

err = ai_multiply_model_create_and_init(

&network,

acts,

NULL);

if(err.type != AI_ERROR_NONE)

{

Error_Handler();

}

ai_input = ai_multiply_model_inputs_get(network,NULL);

ai_output = ai_multiply_model_outputs_get(network,NULL);

}

Call this once in main() after HAL_Init().

## AI Prediction Function

float AI_Predict(float input)

{

ai_i32 batches;

/* Fill input tensor */

in_data[0] = input;

ai_input[0].n_batches = 1;

ai_output[0].n_batches = 1;

ai_input[0].data = AI_HANDLE_PTR(in_data);

ai_output[0].data = AI_HANDLE_PTR(out_data);

batches = ai_multiply_model_run(

network,

ai_input,

ai_output);

if(batches != 1)

{

Error_Handler();

}

return out_data[0];

}

## Main Function

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

AI_Init();

float test_values[] =

{

1,

2,

5,

7,

10,

15

};

for(int i=0;i<6;i++)

{

float y = AI_Predict(test_values[i]);

printf("%.2f --> %.2f\r\n",

test_values[i],

y);

}

printf("Input : %.2f\r\n",x);

printf("Output: %.2f\r\n",y);

while(1)

{

}

}

Expected output

Output

1.00 --> 2.01

2.00 --> 3.99

5.00 --> 10.02

7.00 --> 13.98

10.00 --> 20.03

15.00 --> 29.96

(or very close to actual value depending on training).

# If You Used Normalization During Training

Suppose Colab used

X = X / 20

Then STM32 must do

float x = 7;

float input = x / 20.0f;

float output = AI_Predict(input);

If the model output is also normalized, de-normalize after inference using the same scale used during training. If you trained directly on raw outputs (e.g., Y = 2X), no output scaling is needed.