Yes — you absolutely can use Machine Learning for Adaptive Cruise Control (ACC) with ultrasonic sensors, and it’s actually a very good teaching + prototype use-case 👍
But the right way to do it (especially for PG-DESD / embedded) is important.

I’ll explain this in 3 layers:

- Reality check (industry vs embedded lab)

- Where ML fits correctly with ultrasonic sensors

- A concrete, implementable example you can actually build on STM32 / ESP32

1️⃣ Reality Check (Very Important)

🚗 In production vehicles

- ACC uses:

  - Radar (77 GHz)

  - Camera

  - Sometimes LiDAR

- Ultrasonic sensors are NOT used for highway ACC

  - Range is short (≤4–5 m)

  - Low update rate

  - No velocity info

👉 So ultrasonic ≠ real highway ACC

🧪 But for low-speed / demo / lab ACC

Ultrasonic sensors are perfect for:

- Stop-and-go ACC

- Indoor robot / golf cart

- Smart trolley

- Mini EV / AGV

- Teaching embedded ML

✔ This is academically and practically valid

2️⃣ Where Machine Learning Fits (Correctly)

❌ What NOT to do

Do not use ML just to replace this:

if (distance < threshold)

brake();

That’s a waste of ML.

✅ What ML SHOULD do in ACC

ML is useful when:

- Sensor data is noisy

- You want smooth, adaptive behavior

- You want to learn driver-like control

Typical ML roles:

- Distance trend prediction

- Risk classification

- Speed command generation

3️⃣ Embedded-Friendly ACC Architecture (Ultrasonic + ML)

🔧 Sensors

- Ultrasonic distance (cm)

- Optional: wheel speed (encoder)

🧠 ML Model Input (example)

[ current_distance,

relative_distance_change,

ego_speed ]

🎯 ML Output

- Desired acceleration / deceleration class

  - Brake hard

  - Brake soft

  - Maintain speed

  - Accelerate

This is a classification problem → perfect for TinyML.

4️⃣ Concrete Example You CAN Implement (Recommended)

📌 Example: Low-Speed Adaptive Cruise Control (LS-ACC)

Use case

Maintain safe distance from the vehicle in front at speeds < 20 km/h

🧠 ML Problem Definition

Input features (3)

| Feature | Source |
| --- | --- |
| Distance (cm) | Ultrasonic |
| Δ Distance (cm/s) | Computed |
| Speed (km/h) | Encoder |

Output classes (4)

| Class | Action |
| --- | --- |
| 0 | Hard Brake |
| 1 | Soft Brake |
| 2 | Maintain |
| 3 | Accelerate |

📊 Training Data (Simple & Teachable)

You can generate synthetic data:

distance = np.random.uniform(20, 200)

delta_d = np.random.uniform(-50, 50)

speed = np.random.uniform(0, 20)

Label logic (teacher-friendly):

if distance < 40 and delta_d < 0:

label = HARD_BRAKE

elif distance < 70:

label = SOFT_BRAKE

elif distance < 120:

label = MAINTAIN

else:

label = ACCELERATE

📌 This mimics human driving logic, but ML learns smoother boundaries.

5️⃣ Embedded-Friendly Model (TinyML)

model = tf.keras.Sequential([

tf.keras.layers.Dense(16, activation='relu', input_shape=(3,)),

tf.keras.layers.Dense(8, activation='relu'),

tf.keras.layers.Dense(4, activation='softmax')

])

✔ Runs easily on STM32F4
✔ < 1 KB weights
✔ < 300 MACs

6️⃣ STM32 Runtime Logic (Very Important)

input[0] = distance_cm / 200.0f;

input[1] = delta_d / 100.0f;

input[2] = speed_kmh / 20.0f;

run_ai();

switch(predicted_class) {

case HARD_BRAKE: motor_pwm = 0; break;

case SOFT_BRAKE: motor_pwm -= 20; break;

case MAINTAIN:   motor_pwm = keep; break;

case ACCELERATE: motor_pwm += 10; break;

}

⚠️ ML suggests, classical control enforces safety

7️⃣ Why This Is a GREAT Teaching Example

✔ Shows ML + control coexistence
✔ Uses cheap sensors
✔ Demonstrates TinyML on STM32
✔ Scales to:

- Radar later

- Camera later

- Sensor fusion later

8️⃣ What to Say If Someone Challenges You

“Ultrasonic sensors are not used for highway ACC, but they are suitable for low-speed adaptive cruise and are excellent for demonstrating embedded ML concepts.”

This is 100% technically correct.

9️⃣ If You Want, I Can Next:

1️⃣ Create a Jupyter notebook for LS-ACC
2️⃣ Generate synthetic ACC dataset
3️⃣ Convert model to X-CUBE-AI
4️⃣ Write STM32 inference + motor control code
5️⃣ Turn this into a PG-DESD lab experiment

Just tell me which one you want next 👍